FLORIDA DEPARTMENT OF TRANSPORTATION

RULE CHAPTER NO.: 	RULE CHAPTER TITLE:

14-61				Florida Turnpike System

RULE NO.:			RULE TITLE:

14-61.0011			Florida Turnpike System

14-61.0012			Definitions

14-61.0013			Tolls

14-61.0014			Limitations on Use of Turnpike System

14-61.0015			Prohibitions on the Turnpike System

14-61.0016			Turnpike Tandem Access

14-61.0017			Other Regulations

14-61.018			Tractor Requirements

14-61.0019			Tire Requirements

14-61.0020			Brake Regulations

14-61.021			Emergency Equipment

14-61.0022			Lead Trailer Requirements

14-61.0023			Converter Dolly Requirements

14-61.024			Lamps, Etc.

14-61.0025			Inspection by Driver

14-61.0026			Coupling Devices/Hitch Connections

14-61.0027			Staging

14-61.0028			Speed Limits, Minimum Distances, Passing, and Operations Under Hazardous Conditions

PURPOSE AND EFFECT:  This is a substantial amendment of Rule Chapter 14-61, consisting of a new Part I, Part II, and Part III structure.  Part I - General will be based upon the current rule.  However, because it will be broken into three separate rules, the existing Rule 14-61.0011 is being repealed and the three new rules, based upon portions of the existing rule, are adopted as new.  Part II - Turnpike Tandems and Part III - Regulations Covering the Operation and Safety of Turnpike Tandems are based upon existing rules in Rule Chapter 14-54, F.A.C.  Upon adoption of these new rules, the existing Rule Chapter 14-54, F.A.C., will be repealed.

SUBJECT AREA TO BE ADDRESSED:  This is a revision of Rule Chapter 14-61, to include repeal of the existing rule and adoption of 17 new rules with a three part structure.  The new rules being adopted in Part I are based upon the existing Rule 14-61.0011, which is being repealed.  The new rules being adopted in Part II and Part III amend and replace existing rules currently in Rule Chapter 14-54, F.A.C.

SPECIFIC AUTHORITY: 316.515(12), 334.044(2), 338.239 FS.

LAW IMPLEMENTED:  316.515, 316.655, 338.22 - 338.241, 338.239 F.S.

IF REQUESTED IN WRITING AND NOT DEEMED UNNECESSARY BY THE AGENCY HEAD, A RULE DEVELOPMENT WORKSHOP WILL BE NOTICED IN THE NEXT AVAILABLE FLORIDA ADMINISTRATIVE WEEKLY.

THE PERSON TO BE CONTACTED REGARDING THE PROPOSED RULE DEVELOPMENT IS:  James C. Myers, Clerk of Agency Proceedings, Florida Department of Transportation, Office of the General Counsel, 605 Suwannee Street, Mail Station 58, Tallahassee, Florida 32399-0458.

THE PRELIMINARY TEXT OF THE PROPOSED RULE DEVELOPMENT IS:

Chapter 14-61 Florida Turnpike System

14-61.0011			Florida Turnpike System (Repealed)

14-61.0012			Definitions

14-61.0013			Tolls

14-61.0014			Limitations on Use of Turnpike System

14-61.0015			Prohibitions on the Turnpike System

14-61.0016			Turnpike Tandem Access

14-61.0017			Other Regulations

14-61.0018			Tractor Requirements

14-61.0019			Tire Requirements

14-61.0020			Brake Regulations

14-61.0021			Emergency Equipment

14-61.0022			Lead Trailer Requirements

14-61.0023			Converter Dolly Requirements

14-61.0024			Lamps, Etc.

14-61.0025			Inspection by Driver

14-61.0026			Coupling Devices/Hitch Connections

14-61.0027			Staging

14-61.0028			Speed Limits, Minimum Distances, Passing, and Operations Under Hazardous Conditions

14-61.0011 Florida=s Turnpike System.

Rulemaking Authority 316.515(12), 334.044(2), 338.239 FS.  Law Implemented 338.01, 338.22 - 338.241 338.239 FS.  History - New 8-5-96, Amended 10-30-96, Repealed          .

14-61.0012 Definitions.

(1) Unless defined below, words, phrases, or terms contained herein shall have the definitions set forth in Florida Statutes, including Chapters 316 and 338, F.S.  As used in these rules and regulations, the following words, phrases, or terms shall have the following meanings, where context will permit:

(a) “Department” means the State of Florida Department of Transportation.

(b) “Turnpike System” means as defined in Section 338.221(6), F.S.

(c) “Turnpike Tandem” means any combination of truck tractor, semitrailer, and trailer combination coupled together so as to operate as a single unit,  in which either the semitrailer or the trailer unit exceeds 28 feet in length but in which neither the semitrailer nor the trailer unit exceeds 48 feet in length and which are operated in compliance with Parts II or III of this rule chapter.

(d) “Turnpike Tandem Permit” means a authorization issued by the Department’s Road Use Permits Office for the specific and limited purpose of allowing combinations known as turnpike tandems to operate on the Turnpike System.

(e) “Tandem Trailer Truck” means as defined in 316.03(71), F.S.

Specific Authority 334.044(2), 338.239 FS.  Law Implemented 316.515, 338.239 FS.  History - New           .

14-61.0013 Tolls.

(1) Vehicle Classifications for Toll Schedule Purposes.  For purposes of determining tolls payable under the Toll Schedule fixed by the Department for use of the Turnpike System, the base fare shall be based on two-axle vehicles and increased by an equal amount for each additional axle.

(2) Toll Rules and Rates on the Florida Turnpike System.

(a) Evasion of Tolls.  This includes entering or leaving the Turnpike System or any part of its right of way, except through the regular toll lanes (except in emergency cases, and then only under the control and supervision of the Florida Highway Patrol or Toll Collectors), or committing any other act with the intent to defraud or evade payment of tolls is prohibited.  Enforcement of toll violations shall be in accordance with Rule Chapter 14-100, F.A.C., and all applicable toll enforcement statutes.

(b) Loss of Toll Ticket. The operator of a vehicle on the ticket system portion of the Turnpike System who, for any reason, does not have a toll ticket upon reaching an exit toll station, shall be charged the toll for the appropriate vehicle classification from the most distant toll station within the closed ticket portion of the Turnpike System.

(c) Exit of Vehicle at Point of Entry.

1. The operator of a vehicle on the ticket system portion of the Turnpike System who presents a toll ticket for payment to a toll collector at the same toll station at which such toll ticket was issued, shall be charged the toll for the appropriate vehicle classification from the nearest legal U-Turn point.

2. The operator of a vehicle on the electronic toll collection portion of the Turnpike System who exits the electronic toll collection portion of the Turnpike System at the same toll station at which such vehicle entered the electronice toll collection portion of the Turnpike System, shall be charged the toll for the appropriate amount for the vehicle classification from the nearest legal U-turn point.

(3) Upon entering the Turnpike System a Turnpike Tandem will be treated as two units and charged according to the current classification schedules and method of toll collection;

(a) The first unit will be the tractor and lead trailer; and

(b) The second unit will be the converter dolly and the second trailer.

Rulemaking Authority 334.044(2), 338.239 FS.  Law Implemented 316.1001, 338.155, 338.165(3), 338.239 FS.  History - New           .

14-61.0014 Limitations on Use of Turnpike System.  In addition to the prohibitions and limitations of Chapters 316 and 338, F.S., use of the Turnpike System and entry thereon by the following is prohibited:

(1) Vehicles, including any load thereon, exceeding the maximum dimensions of Section 316.515, F.S., except under special hauling permit issued by the Department or vehicles operated under Tandem Permits issued by the Department.

(2) Vehicles carrying explosives, except under a special hauling permit issued by the Department, and in compliance with the rules and regulations promulgated by the State Insurance Commissioner and Section 316.302, F.S.

Rulemaking Authority  316.550(5), 334.044(2), 338.239 FS.  Law Implemented 316.550(1), 324.044(14), 338.239 FS.  History - New           .

14-61.0015 Prohibitions on the Turnpike System.

(1) Hitchhiking - Loitering. The solicitation of a ride, commonly known as “hitchhiking,” on any portion of the Turnpike System, including toll plazas, is strictly prohibited. Loitering in or about the toll plazas, bridges, overpasses, underpasses, or any other structure, or any other portion of the Turnpike System, is prohibited.

(2) Soliciting or Carrying on Commercial Activity.  No person shall:

(a) Engage in any commercial activity on the Turnpike System without the written permission of, or unless under contract with, the Department or Turnpike Enterprise. Nor shall any person solicit business or funds for any purpose on the Turnpike System without written permission granted by the Department or Turnpike Enterprise.  No person shall at any time or in any manner electioneer on any part of the Turnpike System for or against any party ticket or any candidate for nomination, or officer on any party ticket, or for or against any proposition of any kind or nature to be voted upon at any election.

(b) Post, distribute, or display signs, advertisement, circulars, printed or written matter on the Turnpike System without written permission from, or written contract with, the Department or Turnpike Enterprise.

(c) Throw, cast, fling, heave, hurl, toss, shoot or discharge any pellet, rock, stone, bomb, gun, firearm or any other article, or item over, across, under or along any road, bridge, overpass, underpass, or any other structure of the Turnpike System.

(d) No person shall disturb, tamper with or attempt to destroy, injure or deface, damage, mutilate, or remove any sign, delineator, structure, building, fence, trees, flowers, shrubs, or any other property or equipment of the Turnpike System, or any of its concessionaires.

(e) Fail, neglect or refuse to comply with the collectors at toll booths and such other officials as may be employed by the Turnpike system for such purposes.

(3) Alcoholic Beverages. The consumption of alcoholic beverages is prohibited on the Turnpike System.

(4) Weapons.  Brandishing of weapons by any person is prohibited on the Turnpike System.

(5) Operation of Vehicles. Except for those provisions which are inconsistent with or modified by these rules, the provisions of Chapter 316, State Uniform Traffic Control, Florida Statutes, shall apply on the Turnpike System.

(6) Speed Limits.  All vehicles shall comply with the posted speed limit.  No vehicles shall be operated on the Turnpike System less than 50 miles per hour, except where a lesser speed is posted, or when necessary to do so under the conditions of the road, inclement weather, or with regard to the actual and potential hazards then existing upon the Turnpike System.

(7) Use of Median Strip.  No person shall operate a vehicle on the median strip.  Driving a vehicle on the median strip is prohibited.

(a) Exceptions.  Prohibition on use of the median strip shall not apply to Turnpike construction vehicles, Florida Highway Patrol, Turnpike Maintenance or official Department vehicles, or to their emergency service vehicles; nor to fire vehicles or ambulances, when operated in the performance of their official duties, provided that the operator thereof uses caution so as not to interfere with or endanger traffic.

(b) Prohibition on use of the median strip shall not apply to other emergency-service vehicles, if the crossing or use of the median strip is necessary for the purpose of towing, repairing or otherwise servicing a disabled vehicle provided that such crossing of the median strip shall be made only under the supervision and with the consent of the Florida Highway Patrol or an employee, agent, or contractor of the Department.  Such crossing or use on the Turnpike System shall be further restricted to emergency service vehicles, not operated by garages under contract with the Department, coming to the assistance of a disabled trucking or bus company vehicle,  provided that such emergency service vehicle is owned and operated by, or under contract with, the subject company whose vehicle is disabled.  Disabled vehicles in tow by any emergency service vehicles operating under these conditions shall be allowed to cross or use the median strip.

(c) Upon the recommendation of the Florida Highway Patrol the Department will authorize parking in the median strip. Parking shall, however, be permitted only if such parking will not interfere with maintenance operations.  Such parking in the median strip shall be authorized only if considered by the Florida Highway Patrol to not be dangerous or impractical.

(8) No U Turns. The making of a U turn at any point on the Turnpike System is prohibited unless authorized by the Florida Highway Patrol or the Department.  Excepted from the provisions of this paragraph are such authorized vehicles as described under paragraph 14-61.015(6) above, and then only under such conditions as are described therein.

(9) Overtaking a Vehicle. The provisions of the Florida Uniform Traffic Control Law shall be applicable to the overtaking and passing of vehicles on the Turnpike System except in areas posted to the contrary.

(10) Parking, Stopping or Standing of Vehicles on Traffic, Deceleration or Acceleration Lanes.  No vehicle on the Turnpike System shall be parked, stopped, or allowed to stand on the traffic lanes, acceleration lanes, deceleration lanes, bridges, structures, access ramps, or on shoulders in front of service areas between the traffic lane and the service area, or at any other place where posted to the contrary.  Parking, standing, or stopping on the shoulders of the Turnpike System shall be permitted only in an emergency, or when authorized by the Department, or as directed by the Florida Highway Patrol, and then only on the shoulder to the right of the traffic lane facing in the direction of travel and only on condition that all wheels and projecting parts of the vehicle and load shall be completely clear of the travel lanes. In the event that it is necessary for the operator of a truck or tractor-trailer to leave a vehicle on the Turnpike System unattended and it is impossible or impractical to have such vehicle towed off the Turnpike System, the operator shall obtain a parking permit from the Florida Highway Patrol before leaving the Turnpike System.  The provisions of this paragraph shall not apply to vehicles owned by the Department.

(11) Impounding of Vehicles.  Vehicles illegally parked or abandoned on the Turnpike System may be towed off the Turnpike System and impounded.  Such vehicles may not be removed from the storage compound until after the payment of towing, storage and other charges.

(12) Penalty.  The penalty provisions of the laws of the State of Florida, and Section 316.655, F.S., where applicable, shall apply to any person violating any of the above rules and regulations.

Rulemaking Authority 334.044(2), 338.2216(1)(b), 338.235, 338.239 FS.  Law Implemented 316.083, 316.090, 316.183, 316.655, 338.234, 338.237, 338.239 FS.  History - New           .

PART II TURNPIKE TANDEMS

14-61.0016 Turnpike Tandem Access.  The Turnpike Enterprise will allow turnpike tandems access to the Turnpike System, consistent with the provisions specified herein:

(1) Size, Weight, and Safety Enforcement.

(a) The Motor Carrier Compliance Office, the Turnpike Enterprise, the Florida Highway patrol, or their respective staffs are authorized to inspect all equipment used in the tandem operation and to reject any defective equipment.

(b) The Motor Carrier Compliance Office has primary responsibility for enforcing commercial vehicle size, weight, and safety laws and rules on the Turnpike System.

(c) The Florida Highway Patrol has primary responsibility for enforcing the State’s general traffic safety on the Turnpike System.

(2) Turnpike Tandem Permits and Certifications.

(a) The Department’s Road Use Permits Office is responsible for issuing oversize/overweight Turnpike Tandem permits.  All other certifications must be obtained from the Turnpike Enterprise.

(b) The permittee is responsible for any vehicle operating with an oversize/overweight permit and other certifications and for complete compliance whth all terms of the permit and certification, including:

1.  Ensuring that the driver is qualified to operate the vehicle and understands the terms and conditions of the permit, certifications, and the provisions of this rule chapter.

2.  Ensuring that the vehicle is inspected and maintained in a safe and reliable condition; and

3. Ensuring that the vehicle operates in conformity with the permit, certifications, and the provisions of this rule chapter.

(c) Turnpike tandem permits are issued for the Turnpike system only.  No authority is given to Turnpike Tandems to travel on routes off the Turnpike System.

(3) Original Application.  To operate Turnpike Tandems on the Turnpike System, submit the following certifications to:

Florida Turnpike Enterprise Director of Highway Operations

Pompano Service Area, M.P. 65

Post Office Box 9828

Fort Lauderdale, Florida 33310-9828.

Form Number	Revision Date	Title

800-040-01	  /09		Certification of Turnpike Tandem Trailer Equipment (Tractor)

800-040-02	  /09		Certification of Turnpike Tandem Trailer Equipment (First or Lead Trailers)

800-040-03	  /09		Certificate of Insurance

800-040-04	  /09		General Certification Covering Turnpike Tandem Trailer Operations by Permittee

800-040-05	  /09		Certification for Special Permit to Operate Turnpike Tandem Trailer Vehicle

800-040-06	  /09		 Certification of Turnpike Tandem Trailer Equipment (Dolly Converters)

800-040-07	  /09		Certification for Special Certificate to Operate Turnpike Tandem Trailer Vehicle

The above listed forms are hereby incorporated by reference and made a part of these rules.  Copies of these forms are available at:

http://formserver.dot.state.fl.us/capture/listings/FormListing.aspx?ListType=FormNumber.

(4) All tractors, laden first semi-trailer, and dollies must be approved and authorized by the Turnpike Enterprise before operating under a Turnpike Tandem certification.  Authorization will be withdrawn by the Turnpike Enterprise when it determines that there is a material inconsistency between the provisions of the certification and the equipment in question, or that the continued operation on the Turnpike System would constitute an unsafe operation.

(5) Identification Numbers and Certification.

(a) An identification number will be issued by the Turnpike Enterprise.  A decal displaying the identification number must be purchased by the Permittee and shall be placed on the left side of each tractor, lead trailer, and dolly approved for use in tandem trailer operations.  The numerals must be white on green background; at least three inches in height; and, must be visible to a person standing at ground level.

(b) The Permittee must complete Form 800-040-01, Certification of Turnpike Tandem Trailer Equipment (Tractor), bearing a description of the tractor.  Upon review and approval by the Department, this certification shall be carried in the cab of the tractor which it describes.  Tractor certificates authorize only the vehicle described therein and shall be made available at any time for inspection by the Turnpike Enterprise, Motor Carrier Compliance Office, Florida Highway Patrol, or their respective staffs.  Any discrepancy between the description on the tractor certificate and the actual description of the vehicle will result in the withdrawal of approval.

(c) The Permittee must complete Form 800-040-02, Certification of Turnpike Tandem Trailer Equipment (First or Lead Trailers), bearing a description of the lead trailer or dolly.  Certificates for lead trailers and dollies are not required to be carried by the operator.

(6) Renewal of General Certification.  Turnpike Tandem certifications may be renewed by submitting Form 800-040-04, General Certification Covering Turnpike Tandem Trailer Operations.  The certificate is renewed and effective September 1st of each year, through August 31st of the following year.  Request for certification renewal must be submitted at least 30 days prior to the expiration date.  The Florida Turnpike Enterprise does not provide notification of certification expiration.

(7) Responsibility of Permittee.

(a) Each certificate to operate turnpike tandems shall be valid only when the Permittee has filed Form 800-040-03, Certificate of Insurance, attesting to the fact that the Permittee has secured public liability insurance maintained in compliance with Sections 627.7415 and 627.742, F.S., and 49 C.F.R., Part 387, Subpart A, where applicable.  The named insured shown on all such applied policies shall include the Florida Highway Patrol, the Motor Carrier Compliance Office, the Turnpike Enterprise, and each of their respective officers, agents, and employees.

(b) Such public liability insurance certificate shall explicitly state that the Turnpike Tandem operations of the Permittee are expressly covered under the policy(ies) in effect, or in the alternative, that there is no exclusion in said policy relative to Turnpike Tandem operations by the Permittee.  Such certificate shall also provide that the coverage under the policy may not be canceled without 30 days prior notice, in writing, to the Executive Director of the Florida Turnpike Enterprise.  In the event of cancellation of such public liability insurance policy, every Turnpike Tandem covered by that certificate shall be automatically cancelled.

(c) Certificates of self-insurance issued by the Florida Department of Highway Safety and Motor Vehicles will be accepted in fulfillment of the insurance requirements stated herein, providing such certificates satisfy all the specific requirements.

(d) Description of coverage shall include: Public liability arising in respect to all movement of tandem trailer units.  This includes service trucks, wreckers, or any other vehicles used in the service of the tandem trailer operation, by the Permittee or by anyone acting by, through, or for the Permitee, including omissions and supervisory acts of the Motor Carrier Compliance office, the Turnpike Enterprise, the Florida Highway Patrol, and each of their respective officers, agents, or employees.

(8) Other Permittee information.  The Permittee shall, upon request, furnish the Turnpike Enterprise with all data and information pertaining to an individual trip by a Turnpike Tandem or the overall tandem trailer operation of the Permittee on the Turnpike System.

(9) Voided Certifications and Permits.  When in the interest of health, safety, or welfare of the citizens of the State, the Turnpike Enterprise determines that operation of a turnpike tandem constitutes a hazard to Turnpike System operations, certifications will be voided in whole or in part.  A turnpike tandem oversize/overweight permit will be voided, in whole or in part, by the Road Use Permits Office or the Motor Carrier Compliance Office if the vehicle is in violation of the requirements of the oversize/overweight permit; or if the operation of the Turnpike Tandem is determined to be unsafe.

Rulemaking Authority 334.044(2), 334.044(14), 338.2216(1)(b), 338.239 FS.  Law Implemented 316.515, 316.646, 321.05, 324.171, 334.044(14), 334.044(32), 338.22 - 338.244, 338.239(2) FS.  History - New           .

PART III - REGULATIONS COVERING THE OPERATION AND SAFETY OF TURNPIKE TANDEMS

14-61.0017 Other Regulations.

(1) Driver Requirements.

(a) All drivers of Turnpike Tandem trailer must have a current Commercial Driver=s License (CDL) with an endorsement for double trailers consistent with the provisions of Section 322.57, F.S.

(b) All drivers of Turnpike Tandems and all other individuals or companies operating turnpike tandems must comply with Section 316.302(1), F.S., except that driver exemptions as set forth in 49 C.F.R. Sections 391.21 and 391.67 shall not apply to drivers of turnpike tandems.

(c) All drivers of Turnpike Tandems must have a minimum of five years= experience driving truck tractor semi-trailer combinations.

(d) A driver of a Turnpike Tandem must have had no suspension or revocation of driving privileges in any state or province during the past three years where such suspension arose out of operations of a commercial motor vehicle.

(2) Overall Length, Height, and Width.

(a) All overdimensional rules of the Turnpike Enterprise shall apply to Turnpike Tandems unless specifically excluded under the provisions of this rule chapter.

(b) The overall cargo carrying length of a Turnpike Tandem, as measured from the front of the first trailer to the rear of the second trailer including, the interval between the two trailers, shall not exceed 106 feet.

(c) Turnpike Tandems shall not exceed 13 feet 6 inches in height or 8 feet 6 inches in width.

(3) Weight and Axle Requirements.

(a) All overweight rules of Section 316.515, F.S., shall apply to Turnpike Tandems unless specifically excluded under the provisions of this rule chapter.

(b) The maximum gross weight of the truck tractor and the first semitrailer of a Turnpike Tandem shall not exceed 80,000 pounds.

(c) The maximum gross weight of the unit of dolly and second trailer of a Turnpike Tandem shall not exceed the lesser of:

1.  67,000 pounds, or

2.  The weight provisions of the State=s outer bridge formula set forth in Section 316.535(5), F.S., as measured between the center of the foremost axle of the dolly and the rearmost axle of the trailer.

(d) In the event that a Turnpike Tandem is composed of trailers of unequal gross weight, the heavier of the two shall be used as the lead trailer.

(e) The gross weight limits described in Chapter 316, F.S., may be exceeded with valid oversize/overweight permit issued by the Road Use Permits Office for a maximum gross weight not to exceed 147,000 pounds.

(f) A minimum of five load bearing axles are required unless stated otherwise in a valid oversize/overweight permit issued by the Road Use Permits Office.

Rulemaking Authority 334.044(2), 338.239 FS.  Law Implemented 316.515(12), 322.57, 322.61, 338.239 FS.  History - New           .

14-61.0018 Tractor Requirements.

(1) A tractor used to propel a Turnpike Tandem shall be capable of traveling at a speed of not less than 50 mph except where lower speed limits are posted.

(2) Prior to approval, both the tractor manufacturer and the Permittee shall certify to the Turnpike Enterprise on Form 800-040-01, Certification of Tandem Trailer Equipment (Tractor), that the vehicles proposed to be furnished and used will meet the minimum speed requirements.

(3) A tractor engaged in Turnpike Tandem operations failing to meet such requirement shall not be used to haul a turnpike tandem on the Turnpike System until the gross loads are reduced, the tractor is modified, or other corrective measures have been taken.

(4) Upon a new certification by both the tractor manufacturer and the Permittee that corrective measures have been taken and the tractor meets the minimum speed requirement, the Turnpike Enterprise will reinstate its approval of the described Turnpike Tandem and it may be used in Turnpike Tandem operations.

(5) A tractor engaged in Turnpike Tandem trailer operations must also certify on Form 800-040-01, Certification of Tandem Trailer Equipment (Tractor), that the unit is qualified to haul a total gross weight of at least 100,000 pounds.

Rulemaking Authority  334.044(2), 338.239 FS.  Law Implemented 316.183(2) FS.  History -           .

14-61.019 Tire Requirements.

(1) Each axle on a Turnpike Tandem must have tires of the same size and construction.  Tires must be properly inflated for the load to be carried.  In no event shall any tire, wheel, or rim exceed the manufacturer=s maximum load-carrying limit.  Tires and tire usage must be consistent with the requirements of 49 C.F.R., Section 393.75, as required by Section 316.302(1), F.S.

(2) A vehicle equipped with dual tires may have the dual tires replaced by a single tire so long as the vehicle, axle, and tire load ratings are not exceeded.

(3) No tire may exceed 550 pounds per inch of tire section width as defined by the rating molded in the tire sidewall.  For example, a designation of 445/50R22.5 designates a tire section width of 445 mm (17.5 inches).

Rulemaking Authority 316.535(1), 334.044(2), 338.239(1) FS.  Law Implemented 316.535(6), 338.239  FS.  History - New          .

14-61.0020 Brake Regulations.

(1) Every Turnpike Tandem shall be equipped with full air brakes or air activated hydraulic brakes on the tractor and either air or electric brakes on the dolly and trailers.  All brakes shall equal or exceed both the equipment requirements and the performance standards cited in Chapter 316, F.S., and Sub-part C “Brakes,” C.F.R., Sections  393.40 through 393.52, hereby incorporated by reference.

(2) The brakes on any vehicle, or combination of vehicles, used in the Turnpike Tandem operations shall be adequate to control the movement of, and to stop and hold, such vehicle, or combination of vehicles, and meet the general requirements of the provisions of the Florida Uniform Traffic Control Law, Section 316.262, F.S.

(3) The Permittee shall certify to the Turnpike Enterprise on Form 800-040-04, General Certification Covering Turnpike Tandem Operations by Permittee, that the brakes on any vehicle or combination of vehicles used as a Turnpike Tandem meet the specific requirements specified in this rule.

Rulemaking  Authority 334.044(2), 338.239  FS.  Law Implemented 316.261 - 316.263, 338.239 FS.  History - New          .

14-61.021 Emergency Equipment.  Each tractor used in Turnpike Tandem trailer operations shall be equipped with the following emergency equipment:

(1) A fire extinguisher having an Underwriters Laboratories rating of 5B:C or two or more fire extinguishers having an Underwriters Laboratories rating of 4B:C or more.  For the purpose of this requirement, a vehicle deemed to be transporting hazardous materials must be placarded in accordance with Section 316.302, F.S., and 49 C.F.R.

(2)  Warning devices for stopped vehicles.  At least three bi-directional reflective triangles which conform to the standard for such devices contained in Section 393.95,Title 49 C.F.R.         Rulemaking Authority 316.302(5), 334.044(2), 338.239  FS.  Law Implemented 316.301, 338.239 FS.  History - New          .

14-61.0022 Lead Trailer Requirements.  Prior to approval, both the trailer manufacturer and the Permittee shall certify to the Turnpike Enterprise on Form 800-040-02, Certification of Tandem Trailer Equipment (First or Lead Trailers) that the vehicle proposed to be furnished will be adequate to meet all requirements of a first semi-trailer.  Semi-trailers operated with this certification must comply with all requirements listed on that form.

Rulemaking Authority 334.044(2), 338.239  FS.  Law Implemented 338.239  FS.  History - New           .

14-61.0023 Converter Dolly Requirements.

(1) A converter (fifth-wheel) dolly used in the Turnpike Tandem operations may have either a single or a double axle, according to its total gross weight.  In addition to the tow bar(s), the dolly vehicle must be equipped with safety chains or cables for connecting the dolly to the lead semi-trailer and adequate to prevent breakaway.

(2) When the distance between the rear of the first semi-trailer and the front of the second semi-trailer is 10 feet or more, the dolly shall be equipped with a device, or the trailers shall be connected along the sides with suitable material, which will advise  other motorists  that the trailers are connected and are in effect one unit.

(3) The Permittee shall certify to the Turnpike Enterprise on Form 800-040-06, Certification of Turnpike Tandem Equipment (Dolly Converters), that the equipment proposed complies with all the requirements listed on that form.

Rulemaking Authority 334.044(2), 338.239 FS.  Law Implemented 338.239FS.  History - New           .

14-61.0024 Lamps, Etc.

(1) Lamps and Reflectors.  Each tractor, trailer, and converter dolly in a  Turnpike Tandem shall be equipped with electrical lamps and reflectors mounted on the vehicle in accordance with Chapter 316, F.S., and Sub-part B “Lighting Devices, Reflectors, and Electrical Equipment,” 49 C.F.R. Sections  393.9 through 393.33, as required by Section 316.302(1),F.S.

(2) Mud Flaps, splash, and spray suppressant devices.  Each Turnpike Tandem shall be equipped with mud flaps and splash and spray suppressant devices meeting the requirements of and mounted on the vehicle in accordance with Section 316.252, F.S.

Rulemaking Authority 316.252, 334.044(2), 338.239  FS.  Law Implemented 316.252, 338.239  FS.  History - New           .

14-61.0025 Inspection by Driver.  After all of the component vehicles in a Turnpike Tandem are completely hooked up and prior to the departure of the unit from the assembly area, the driver or a mechanic shall inspect the tandem unit to ensure that each item is in proper operating condition.  No Turnpike Tandem shall be driven unless the driver thereof shall have satisfied the requirements of 49 C.F.R. Sections 392.7 through 392.9, hereby incorporated by reference, as required by Section 316.302(1), F.S.

Rulemaking Authority 334.044(2), 338.239  FS.  Law Implemented 338.239   FS.  History - New        .

14-61.0026 Coupling Devices/Hitch Connections.

(1) All coupling devices shall equal or exceed both the equipment requirements and the performance standards established in 49, C.F.R. Section 393.70, as required by Section 316.302(1), F.S.

(2) Vehicles in a Turnpike Tandem shall be designed, constructed, and connected as to ensure that shifting or swerving from side to side will not exceed two inches to each side of the path of the towing vehicle when it is moving in a straight line.

(3)  All coupling devices/hitch connections shall be of a no-slack type which must be visible and operating.  All drawbars, pickup plates, and fifth wheels must be rated to exceed the weight carried.  Any kingpin must be rated to exceed the weight carried.  Any kingpin must be solid and must be permanently fastened.

Rulemaking Authority 334.044(2), 338.239 FS.  Law Implemented 338.239  FS.  History           .

14-61.0027 Staging.

(1) Turnpike Tandems shall be made and broken down only in designated Turnpike System staging areas.  All movement across traffic while entering or leaving a staging area shall be made using extreme caution.

(2) Permitees shall assume all responsibility for their vehicles and equipment, as well as the contents thereof, while such vehicles and equipment are in a staging area.

(3) For the purposes of safety and meeting unforeseen local conditions, the Permittees’ use of staging areas is subject to Turnpike Enterprise regulations at the staging areas, including the prohibition of staging during certain hours; temporary suspension of staging; and limitation of the time that equipment may be parked in the staging area.

Rulemaking Authority 334.044(2), 338.239 FS.  Law Implemented 338.239   FS.  History - New           .

14-61.0028 Speed Limits, Minimum Distances, Passing, and Operations under Hazardous Conditions.

(1) Speed Limits.  When the speed of a Turnpike Tandem drops to 20 mph less than the posted maximum speed limit, the driver must use emergency flashers to notify the passing traffic that they are approaching a vehicle traveling substantially slower than the rest of the traffic.

(2) Minimum Distances.  A minimum distance of 100 feet for each 10 mph of speed shall be maintained between a Turnpike Tandem trailer and another vehicle traveling in front of it in the same travel lane, unless weather or other roadway conditions do not permit such distance.

(3) Passing.  A Turnpike Tandem may pass another vehicle traveling in the same direction only if the speed differential will allow the Turnpike Tandem to complete the maneuver and return to the normal driving lane within a distance of one mile and be performed within the posted speed limit. Turnpike Tandems must stay in the right lane, or those lanes designated for travel by posted signs, unless they are in the act of passing.

(4) Operations under Hazardous Conditions.

(a) Drivers of Turnpike Tandems shall exercise extreme caution when hazardous conditions exist, such as fog, smoke, dust, mist, or rain.  Speed shall be reduced when such conditions exist.

(b) When hazardous weather conditions become dangerous, the driver or Permittee shall discontinue operations, and such operations shall not be resumed until the vehicle can be safely operated.  The Turnpike Enterprise, Motor Carier Compliance Office, Florida Highway Patrol, or their respective staffs may restrict or prohibit operations during periods when traffic, weather, or other safety conditions make such operations unsafe or inadvisable.

Rulemaking Authority 334.044(2), 338.239FS.  Law Implemented 316.55, 338.239  FS.  History - New           .